[Your logo]

Risk assessment and method statement

# Cleaning RAMS

Commercial cleaning, window cleaning and jet washing. Pre-filled with typical hazards, controls and sequence: edit it for this site.

Management of Health and Safety at Work Regulations 1999, regulation 3. COSHH Regulations 2002.

| Detail | Entry |
| --- | --- |
| Project |  |
| Site address |  |
| Activity | Commercial cleaning, window cleaning and jet washing |
| RAMS reference |  |
| Contractor carrying out the work |  |
| Principal contractor |  |
| Prepared by |  |
| Approved by |  |
| Revision |  |
| Date |  |

Revision record. Reissue and rebrief when the sequence, the programme, the team or the conditions change.

| Rev | Date | What changed | By |
| --- | --- | --- | --- |

THE WORK

# The activity, the place and the people

## Scope of the activity

Cleaning of [building / areas] to the specification [ref], including [general and floor cleaning / washrooms / internal and external glazing / jet washing of (surfaces) / builders' clean]. Window cleaning by [water fed pole / ladder / MEWP]. Not included: [list any exclusions].

## Location and access

Access via [entrance], with keys or passes from [contact]. Working hours [18:00 to 22:00]. Products and equipment stored in [store]. Water and power from [points]. Occupied areas: [details], with a dry route kept open.

## Interfaces with other trades

Client contact: [name], for access, alarms, times and occupied areas. On construction sites, the principal contractor's rules, induction and handover of areas for cleaning. Other trades: [trades], kept clear of wet floors and exclusion zones.

## Documents this RAMS relies on

Specification [ref]. Safety data sheets and COSHH assessments for each product. Site rules or construction phase plan. Lone working procedure and check-in contacts. Equipment inspection records.

## Programme

| Start date | Duration | Working hours |
| --- | --- | --- |

## The team, and their competence

| Role | Name | Card, ticket or training needed | Contact |
| --- | --- | --- | --- |
| Cleaners |  | Trained in the products, dilutions, equipment and this RAMS before starting, with new starters supervised. |  |
| Supervisor |  | Experienced cleaner able to check work, brief the team and act on a missed check-in. |  |
| Window cleaners |  | Trained in water fed pole use and safe ladder use; ladder work only by those trained for it. |  |
| Pressure washer operators |  | Trained on the machine, its pressure settings and the exclusion zone. |  |
| MEWP operators |  | IPAF PAL card for the category of machine. |  |
| RPE wearers |  | Face fit tested for tight-fitting masks, and clean shaven where the mask seals. |  |

METHOD STATEMENT

# The sequence of work

The order the team will work in on the day, with the controls built into each stage. If the sequence changes on site, revise the RAMS and brief it again.

| Stage | What is done, and how | Controls at this stage | By |
| --- | --- | --- | --- |
| Before the work | Visit the site, agree the specification, times and access with the client, and check the windows, roofs, power, water and any lone or out of hours work. Collect the safety data sheets for every product. | COSHH assessments for each product; access method chosen to avoid work at height; RAMS agreed with the client. | Contracts manager |
| Arrival and briefing | Sign in, collect keys or passes, attend any site induction, then brief the team on this RAMS, the products in use and the check-in routine. | Briefing signed against this revision; training records checked; equipment checked before use. | Supervisor |
| Set up the work area | Fit barriers and signs, plan a dry route for occupants, connect power and water, and lay leads and hoses clear of walkways. | RCD protection on mains equipment; leads and hoses run along walls; barriers rather than signs alone. | Cleaners |
| General and floor cleaning | Clean from high to low and from the far end towards the exit, working in sections so people can pass. Spot clean spills as they are found. | Products at the label dilution; dry mop or squeegee after wet cleaning; floors dry before barriers come down. | Cleaners |
| Washrooms, bins and waste | Clean washrooms and empty bins, checking bags and bins for needles and broken glass before handling them. | Bleach kept apart from acid cleaners; tongs and sharps box to hand; spill kit for body fluids. | Cleaners |
| Windows from the ground | Clean external glass with water fed poles from the ground, and internal glass from the inside, before any work at height is considered. | Overhead lines checked before poles are raised; poles lowered to move; breaks taken. | Window cleaners |
| High-level work | Where the ground cannot be used, work from a tied or footed ladder for short, light tasks, or from a MEWP or tower for longer ones. | Ladders checked and secured, other equipment past 30 minutes at a time; MEWP by an IPAF operator; fragile surfaces kept off. | Window cleaners |
| Jet washing | Set the exclusion zone, check the surface for bird droppings, loose material and electrical fittings, then wash from the top down, keeping the jet away from people. | Trained operator; visor and waterproofs; no pressure washing of droppings; run-off kept out of surface water drains. | Operator |
| Builders' cleans | Clean only the areas the site manager has handed over, removing dust by vacuum or damp methods, and glass, frames and fittings last. | Site rules and induction followed; M class vacuum, no dry sweeping; no entry beyond barriers. | Supervisor |
| Clear up and hand back | Clean and store equipment, lock products away, remove waste, check floors are dry, secure the building and check out. | Products returned to a locked store; building secured as agreed; final check-in made. | Supervisor |

RISK ASSESSMENT

# Hazards, controls and the risk that remains

Each significant hazard rated before and after its controls, as likelihood (1 to 5) times severity (1 to 5). Bands: 1 to 4 low, 5 to 12 medium, 15 to 25 high. The ratings are typical: check them for this site.

| Hazard | Who could be harmed, and how | Risk before | Controls | Risk after | By |
| --- | --- | --- | --- | --- | --- |
| Corrosive and irritant cleaning products | Cleaners diluting, decanting and using products, from burns to the skin and eyes. | 3 x 4 = 12 Medium | • A COSHH assessment for each hazardous product, from its label and safety data sheet; milder products used where they do the job. • Products used at the dilution on the label, from dosing equipment where supplied, and decanted only into labelled containers. • Gloves and eye protection as the data sheet specifies; eyewash on hand where corrosives are used. | 1 x 4 = 4 Low | Supervisor |
| Mixing products and toxic gas | Cleaners and occupants, from gas released when bleach meets acid or other products. | 3 x 4 = 12 Medium | • Products never mixed. Bleach is kept apart from acid descalers and toilet cleaners, which release chlorine gas on contact. • Incompatible products stored separately, in their original containers, and cupboards kept locked. • Rooms ventilated while products are in use; anyone affected moved to fresh air. | 1 x 4 = 4 Low |  |
| Wet work and dermatitis | Cleaners whose hands are wet for long periods or many times a day. | 4 x 2 = 8 Medium | • Gloves suited to the product, with cotton liners for long wear, and changed when wet inside. • Contamination removed promptly, hands washed and dried thoroughly, and skin cream used regularly. • Regular skin checks by a responsible person, with any redness or cracking reported early. | 2 x 2 = 4 Low |  |
| Slips on wet and contaminated floors | Cleaners, occupants and the public walking on floors left wet or being cleaned. | 4 x 3 = 12 Medium | • Access stopped with barriers, locked doors or by cleaning in sections, keeping a dry route open. Signs and cones only warn. • Floors cleaned when they are quietest, with a dry mop or squeegee to cut drying time; spills spot cleaned straight away. • Slip-resistant footwear for cleaners; signs removed once the floor is dry. | 2 x 3 = 6 Medium | Supervisor |
| Falls from ladders and stepladders | Window cleaners and cleaners reaching high-level glass, fittings and ledges. | 4 x 4 = 16 High | • Water fed poles or work from the inside first; ladders only for light, low-risk work, with other equipment considered for any task over 30 minutes at a time. • Ladders checked before use, footed or tied whenever possible, never on a wet or uneven base. • Stepladders face on, locks engaged, three points of contact; no overreaching. | 2 x 4 = 8 Medium |  |
| Water fed poles near power lines | Window cleaners extending poles near overhead lines, cables and building services. | 2 x 5 = 10 Medium | • Overhead lines and cables checked before poles are extended; no pole work within reach of them. • Poles carried lowered and extended only at the point of work. | 1 x 5 = 5 Medium |  |
| Roofs, gutters and fragile surfaces | Cleaners clearing gutters, roof glazing and high-level surfaces. | 3 x 5 = 15 High | • Gutters cleared from the ground with a gutter vacuum and poles where possible. • No one on a roof unless the plan provides edge protection or a MEWP; fragile rooflights and sheets covered or kept off. • MEWPs used only by IPAF trained operators, with a restraint lanyard in a boom. | 1 x 5 = 5 Medium |  |
| Pressure washer jet injuries | Operators, helpers and passers-by, from cuts and water forced under the skin. | 3 x 4 = 12 Medium | • Only trained operators; lance never pointed at people, trigger lock on when not spraying. • Exclusion zone barriered, with the work timed or the route closed so the public are kept clear of spray and debris. • Face visor, waterproof clothing and boots; any jet injury seen by a doctor at once, however small it looks. | 1 x 4 = 4 Low | Supervisor |
| Electricity and water | Operators of electric pressure washers, scrubber dryers and vacuums in wet areas. | 3 x 5 = 15 High | • Equipment and leads checked each time before use, with a set testing interval suited to wet and outdoor use. • Mains equipment on an RCD-protected supply, or reduced voltage on site; plugs and connections kept out of water. • Damaged equipment taken out of use and labelled. | 1 x 5 = 5 Medium |  |
| Petrol and diesel washers and fuel | Operators and occupants, from exhaust fumes, hot surfaces and fuel spills. | 2 x 5 = 10 Medium | • Engine-driven units run only in the open air, never in buildings, car parks or enclosed yards. • Refuelled with the engine off and cool, from a proper fuel can, away from drains. | 1 x 5 = 5 Medium |  |
| Needles and sharps | Cleaners emptying bins, clearing washrooms, stairwells, car parks and grounds. | 3 x 4 = 12 Medium | • Bins and bags never compacted by hand or foot; bags held away from the body. • Needles picked up only with tongs or a litter picker, into a sharps box, wearing puncture-resistant gloves. • Wound encouraged to bleed, washed with soap and water, and urgent medical advice sought. | 1 x 4 = 4 Low |  |
| Body fluids and contaminated waste | Cleaners dealing with blood, vomit and soiled waste in washrooms and public areas. | 3 x 3 = 9 Medium | • Spill kit with disposable gloves, apron and absorbent; spill contained, then the area disinfected to the product label. • Cuts covered with waterproof dressings; hands washed after removing gloves. | 1 x 3 = 3 Low |  |
| Bird droppings | Cleaners clearing droppings from ledges, canopies, gutters and plant areas. | 3 x 4 = 12 Medium | • Droppings damped down first and removed by hand tools into sealed bags. Never pressure washed, which creates contaminated droplets. • RPE with an assigned protection factor of 20, such as FFP3, face fit tested; disposable overalls and gloves. • Hands washed before eating or smoking; breaks away from the work area; no one with a weakened immune system on this task. | 1 x 4 = 4 Low |  |
| Lone and out of hours working | Cleaners working alone early, late or at weekends, in empty or remote buildings. | 3 x 4 = 12 Medium | • Lone work agreed in the risk assessment, with check-ins at arrival, set times and departure, and action on a missed one. • No higher-risk tasks, such as ladder work or jet washing, done alone. • Alarm, entry and exit procedures agreed with the client; doors kept secured while working. | 2 x 4 = 8 Medium | Supervisor |
| Manual handling and repetitive work | Cleaners lifting buckets, machines and waste, and using poles and mops for long periods. | 4 x 2 = 8 Medium | • Buckets filled and emptied where they stand, with trolleys and wheeled buckets; machines moved by two on stairs. • Lightweight poles of the right length, with tasks rotated and breaks taken. • Waste bags kept to a weight that can be lifted easily. | 2 x 2 = 4 Low |  |
| Floor machines and trailing cables | Operators and occupants, from trips over leads and contact with rotating brushes. | 3 x 2 = 6 Medium | • Leads run along walls and kept behind the operator; cordless machines where practicable. • Machines switched off and unplugged before brushes or pads are changed. | 1 x 2 = 2 Low |  |
| Dust on builders' cleans | Cleaners clearing plaster, concrete and other construction dust at the end of a job. | 4 x 3 = 12 Medium | • Dust removed with an M class vacuum or by damp methods, never by dry sweeping or blowing. • FFP3 RPE, face fit tested, for dusty work. | 2 x 3 = 6 Medium |  |
| Unfinished buildings and other trades | Cleaners on site near open edges, unfinished stairs, live services and plant. | 3 x 4 = 12 Medium | • Site induction and the principal contractor's rules followed; areas cleaned only when handed over for cleaning. • No entry beyond barriers or into unguarded areas; damage or hazards reported, not moved. • Helmet, boots and high visibility clothing to the site standard. | 1 x 4 = 4 Low |  |

MHSWR regulation 3(6): with five or more employees, record the significant findings.

EQUIPMENT

# Plant, equipment and PPE

| Item | Checks and certificates | Operator competence |
| --- | --- | --- |
| Water fed pole system | Pole, hose and joints checked before use; lightweight pole for the height. | Trained user |
| Ladders and stepladders | Pre-use check each day; detailed inspection on a set schedule. | Trained user |
| Pressure washer | Lance, trigger lock, hoses and leads checked before use; RCD tested. | Trained operator |
| Scrubber dryer and vacuums | Leads, plugs and brushes checked; M class vacuum for construction dust. | Trained user |
| MEWP | Pre-use check; thorough examination at least every 6 months. | IPAF PAL card |
| Spill and sharps kits | Stocked and in date; sharps box sealed when full. | Each cleaner |

## Personal protective equipment

PPE is the last line of defence, after the controls above. Specify the type, not just the item.

| Item | Needed | Type and specification |
| --- | --- | --- |
| Safety helmet | Yes | Where site rules require |
| Safety footwear | Yes | Slip-resistant; waterproof boots for jet washing |
| High-visibility clothing | Yes | On sites and near traffic |
| Gloves | Yes | As the data sheet specifies; puncture-resistant |
| Eye protection | Yes | Decanting and spraying; visor for jet washing |
| Hearing protection |  |  |
| Respiratory protection | Yes | FFP3 for construction dust and droppings |
| Harness or restraint |  |  |

CONTROLS

# Substances, permits, emergencies and the environment

## Hazardous substances

| Substance | COSHH assessment reference | Controls |
| --- | --- | --- |
| Bleach and chlorine-based cleaners | [ref] | Never mixed with acids or other products; label dilution; gloves and eye protection. |
| Acid descalers and toilet cleaners | [ref] | Kept apart from bleach; gloves and eye protection; ventilation. |
| Detergents and degreasers | [ref] | Label dilution from dosing equipment; gloves for long or repeated contact. |
| Construction dust on builders' cleans | [ref] | M class vacuum or damp methods, no dry sweeping; FFP3 RPE. |

## Permits and plans

| Permit or plan | Needed | Reference |
| --- | --- | --- |
| Hot work permit |  |  |
| Permit to work | Yes |  |
| Permit to dig or break ground |  |  |
| Confined space entry |  |  |
| Lift plan |  |  |
| Temporary works design |  |  |

## Emergency and rescue

First aider and first aid kit on site or with the team, with eyewash for chemical splashes. Anyone affected by fumes is moved to fresh air. Needlestick or jet injuries get urgent medical attention. Missed check-ins are acted on at once. Nearest accident and emergency department named on the briefing.

## Environment and neighbours

No cleaning products, wash water or jet washing run-off into surface water drains or watercourses. Product kept to the amount needed. Waste, sharps and contaminated material bagged and removed through the agreed waste route. Noise kept to agreed hours.

PRODUCTS REGISTER

# Cleaning products register

List every product used on the contract, from its label and safety data sheet. Add a row before anything new is used, and brief the team on it.

| Product | Used for | Hazards from the data sheet | Dilution | Never mix with | Gloves and eyes |
| --- | --- | --- | --- | --- | --- |
| Bleach or chlorine cleaner [name] | Washroom disinfection | [From section 2 of the data sheet] | [As label] | Acids, descalers, toilet cleaners, other products | Gloves; eye protection when decanting |
| Acid descaler [name] | Limescale, toilets, urinals | [From section 2 of the data sheet] | [As label] | Bleach and chlorine products | Gloves; eye protection |
| General purpose detergent [name] | Surfaces and floors | [From section 2 of the data sheet] | [As label] | Other products | Gloves for long contact |
| Glass cleaner [name] | Internal glazing | [From section 2 of the data sheet] | [As label] | Other products | [As data sheet] |
| Degreaser [name] | Plant, bin stores, hard standing | [From section 2 of the data sheet] | [As label] | Other products | Gloves; eye protection |

COSHH Regulations 2002, regulations 6 and 7; HSE, COSHH and cleaners: key messages.

BRIEFING RECORD

# Briefed before the work starts

Each person signs against this revision, on the day, before starting. Anyone joining later is briefed before they join the work. When the RAMS is revised, everyone is briefed again.

| Briefed by | Revision briefed | Date and time |
| --- | --- | --- |

| Name | Company | Card or ticket number | Signature | Date |
| --- | --- | --- | --- | --- |