[Your logo]

Risk assessment and method statement

# Demolition and strip out RAMS

Soft strip and demolition of a structure. Pre-filled with typical hazards, controls and sequence: edit it for this site.

Management of Health and Safety at Work Regulations 1999, regulation 3. CDM 2015, regulation 20.

| Detail | Entry |
| --- | --- |
| Project |  |
| Site address |  |
| Activity | Soft strip and demolition of a structure |
| RAMS reference |  |
| Contractor carrying out the work |  |
| Principal contractor |  |
| Prepared by |  |
| Approved by |  |
| Revision |  |
| Date |  |

Revision record. Reissue and rebrief when the sequence, the programme, the team or the conditions change.

| Rev | Date | What changed | By |
| --- | --- | --- | --- |

THE WORK

# The activity, the place and the people

## Scope of the activity

Soft strip and [full / partial] demolition of [structure] as shown on [drawing ref], down to [ground level / slab / foundations]. Includes service isolations, removal of arisings and [below-ground works]. Asbestos removal by [licensed contractor]. Not included: [list any exclusions].

## Location and access

Site access via [gate]. Hoarding on [boundaries]. Vehicle route [route], kept apart from the pedestrian route by barriers. Exclusion and debris zones as shown on [plan ref]. Footpath on [street] [kept open / diverted]. Working hours [08:00 to 17:00].

## Interfaces with other trades

Licensed asbestos contractor: [name], works complete before structural demolition. Service owners: [names], for disconnections. Neighbours and adjoining owners: [details]. Other trades on site: [trades], kept outside the zones.

## Documents this RAMS relies on

Refurbishment and demolition survey [ref] and clearance certificates. Structural survey and demolition engineer advice [ref]. Local authority notice [ref]. Disconnection confirmations. Temporary works design [ref]. Construction phase plan.

## Programme

| Start date | Duration | Working hours |
| --- | --- | --- |

## The team, and their competence

| Role | Name | Card, ticket or training needed | Contact |
| --- | --- | --- | --- |
| Demolition operatives |  | CCDO card for the role, from the NFDC scheme administered by the NDTG. |  |
| Supervisor |  | CCDO Supervisor card, on site throughout. |  |
| Method statement author |  | A competent person, with a demolition engineer for complex structures. |  |
| Plant operators |  | CPCS or NPORS card for the high reach, excavator or loading shovel and its attachment. |  |
| Asbestos removal |  | A contractor holding an HSE licence for licensable work. |  |
| Hot cutting |  | Trained in the use of oxy-fuel equipment and the hot work permit. |  |

METHOD STATEMENT

# The sequence of work

The order the team will work in on the day, with the controls built into each stage. If the sequence changes on site, revise the RAMS and brief it again.

| Stage | What is done, and how | Controls at this stage | By |
| --- | --- | --- | --- |
| Before the day | Review the pre-construction information, the asbestos survey, the structural survey and the services. Confirm the local authority notice (England and Wales), the method and the exclusion and debris zones. | Written demolition plan by a competent person; RAMS approved by the principal contractor. | Contracts manager |
| Arrival and briefing | Sign in, attend the induction, then brief the team on this RAMS, the sequence, the zones, the asbestos findings and the emergency plan. Check plant and cards. | Briefing signed against this revision; CCDO and plant cards checked; plant pre-use checks recorded. | Supervisor |
| Secure the site | Put up hoarding, set out vehicle and pedestrian routes, and barrier the exclusion and debris zones. | Public kept out of the debris zone; footpaths diverted or protected; banksman at the gate. | Supervisor |
| Isolate services | Have each service disconnected by its owner, prove the structure dead, and set up any temporary supplies the work needs. | Written disconnection confirmations on file; cables and pipes treated as live until proven dead. | Supervisor |
| Asbestos removal | The asbestos identified in the survey is removed under its own plan of work, by a licensed contractor where the work is licensable, before other major works start. | Clearance certificate for each area of licensed work before our team goes in; suspect finds stop the work. | Asbestos contractor |
| Soft strip | Strip fixtures, fittings, ceilings, partitions, glass and finishes, sorting materials for reuse and recycling, and leave the structure bare. | Openings covered; debris by chute or skip; sharps and hazardous finds dealt with first. | Supervisor |
| Structural demolition | Take down the structure in the planned order, usually from the top down, by machine where practicable, keeping what remains stable at every stage. | Nobody in the exclusion or debris zones; water suppression; sequence changes agreed with the engineer. | Supervisor |
| Process and remove | Sort, process and load the arisings for removal or reuse on site, with lorries loaded inside the site. | Crusher and loading zones barriered; dust suppressed; waste transfer notes for every load. | Supervisor |
| Below ground | Break out slabs and foundations to the depth in the scope, then fill or make safe basements, voids and excavations. | Buried services located first; basement walls propped where they hold back ground; edges fenced. | Supervisor |
| Clear and hand back | Clear the site to the agreed level, remove plant and barriers, and hand the site back with the records. | Site walked with the principal contractor or client before the team leaves. | Contracts manager |

RISK ASSESSMENT

# Hazards, controls and the risk that remains

Each significant hazard rated before and after its controls, as likelihood (1 to 5) times severity (1 to 5). Bands: 1 to 4 low, 5 to 12 medium, 15 to 25 high. The ratings are typical: check them for this site.

| Hazard | Who could be harmed, and how | Risk before | Controls | Risk after | By |
| --- | --- | --- | --- | --- | --- |
| Premature or uncontrolled collapse | Operatives, plant operators, neighbours and the public, if the structure falls before or beyond the plan. | 3 x 5 = 15 High | • Structural survey of the form, load paths, floors, shear walls, stairs and lift shafts before the method is written, with a demolition engineer where needed. • Demolition sequence keeps the remaining structure stable at every stage, with temporary propping or restraint to a temporary works design. • Any change from the planned sequence agreed with the engineer and the RAMS revised before work carries on. | 1 x 5 = 5 Medium | Contracts manager |
| Falls from edges, floors and roofs | Operatives working at open edges, on upper floors and on roofs during strip out and demolition. | 4 x 5 = 20 High | • Machine demolition from the ground, or remote controlled machines, in place of people working at height wherever practicable. • Edge protection at open edges, with a top guard-rail at least 950mm high and no gap over 470mm, or MEWPs for work at height. • Harness and lanyard only where collective protection cannot be provided, with a rescue plan. | 1 x 5 = 5 Medium |  |
| Falls through floors, roofs and openings | Operatives on fragile roofs, rotten or weakened floors, and near stair and lift openings. | 3 x 5 = 15 High | • Fragile roofs, rooflights and weakened floors identified in the survey and marked before anyone goes near them. • Openings covered with fixed, marked covers or guarded; nobody walks on a roof or floor not confirmed safe. • Debris kept off upper floors so it does not overload them. | 1 x 5 = 5 Medium |  |
| Asbestos found or disturbed | Operatives, other trades and neighbours, from fibres released during strip out or demolition. | 4 x 5 = 20 High | • Refurbishment and demolition survey to HSG264 for the whole building before work, read and briefed. • Asbestos removed by a licensed contractor where the work is licensable, with a clearance certificate, before other major works start. • Suspect material found later means stop work, keep people out and report it; work restarts only once it is sampled and dealt with. | 1 x 5 = 5 Medium | Contracts manager |
| Live services in the structure | Operatives and plant operators, from electric shock, gas escape, fire or flooding. | 3 x 5 = 15 High | • Electricity, gas, water and telecoms disconnected by the owners, with written confirmation, before demolition starts. • Cables and pipes treated as live until proven dead; temporary supplies for the work run separately and marked. • Buried services under slabs and in the ground located before breaking out below ground level. | 1 x 5 = 5 Medium | Supervisor |
| Overhead power lines | High reach and excavator operators, and anyone near plant or a structure that falls towards a line. | 2 x 5 = 10 Medium | • Lines identified before work and the network operator asked to divert or switch them off where plant or debris could reach them. • Where lines stay live, barriers and goal posts keep plant out of reach, and the sequence drops debris away from the line. | 1 x 5 = 5 Medium |  |
| Falling debris and materials | Operatives, other trades and the public below and around the work. | 4 x 5 = 20 High | • Exclusion and debris zones set out and barriered, sized for the method and the height, with nobody inside them while demolition is live. • Debris lowered by chute, skip or machine, never thrown; chutes enclosed and the base area fenced. • Scaffold and netting used to contain debris only where designed for that load by a competent person. | 1 x 5 = 5 Medium |  |
| Demolition plant: overturning and struck by | Plant operators, banksmen and operatives near high reach machines and excavators. | 3 x 5 = 15 High | • Machine chosen for the reach and the work, with a cab guard against falling material, and set up on firm, level ground or a prepared platform. • Exclusion zone around each machine; nobody approaches until the operator has seen them and stopped. • Operators hold a CPCS or NPORS card for the machine, and are trained on the attachments fitted. | 1 x 5 = 5 Medium |  |
| Vehicles and people on site | Operatives, drivers and visitors near muck-away lorries, loading shovels and dumpers. | 4 x 5 = 20 High | • Vehicle and pedestrian routes planned and kept apart by barriers, with a one-way system where space allows. • Trained banksman for every reversing movement and at the site gate. | 1 x 5 = 5 Medium | Supervisor |
| Adjoining buildings and party walls | Neighbours and occupants of attached buildings, and operatives near a shared wall. | 3 x 4 = 12 Medium | • Condition of adjoining buildings recorded before work, and party walls and shared structure assessed by the engineer. • Shared walls propped or restrained to the temporary works design before the structure tying them in is removed. • Weather protection for exposed party walls agreed with the neighbours before work starts. | 1 x 4 = 4 Low |  |
| The public and neighbours | Passers-by, neighbours and children who could enter the site or the debris zone. | 3 x 5 = 15 High | • Site enclosed by hoarding or fencing that keeps the public out of the debris zone, secured at the end of each day. • Footpaths diverted or protected where the debris zone reaches them, with any licence the highway authority requires. | 1 x 5 = 5 Medium |  |
| Silica and demolition dust | Operatives breaking, crushing and cutting concrete and masonry, and neighbours downwind. | 4 x 3 = 12 Medium | • Water suppression at the point of demolition and on crushers and processing. • Machines with filtered, sealed cabs; on-tool extraction with an H or M class unit for hand-held cutting. • RPE with an assigned protection factor of 20, face fit tested, for anyone working in dust outside a cab. | 2 x 3 = 6 Medium |  |
| Noise and hand-arm vibration | Operatives using breakers, saws and hand tools, and anyone near crushers and machine breakers. | 4 x 3 = 12 Medium | • Machine-mounted or remote controlled breakers in place of hand-held ones wherever practicable. • Vibration exposure planned per tool and per person against the action value of 2.5 m/s² A(8). • Hearing protection worn wherever exposure can reach 85 dB(A), the upper exposure action value. | 2 x 3 = 6 Medium |  |
| Hot cutting, fire and fume | Operatives cutting steelwork, and anyone near combustible materials, painted steel or fuel. | 3 x 4 = 12 Medium | • Hot work permit, combustibles cleared, extinguishers at hand and a fire watch during and after cutting. • Old paint on steelwork checked for lead before burning, with cold cutting or shears used where it is. • Gas cylinders stored upright, secured and away from the demolition, with flashback arrestors fitted. | 1 x 4 = 4 Low |  |
| Hazards in derelict buildings | Operatives in the soft strip meeting needles, droppings, chemicals, rot or vermin. | 3 x 3 = 9 Medium | • Pre-start walk round to find sharps, bird droppings, chemicals, fuel and stored waste, with specialists for any of them. • Sharps picked up with tools into a sharps bin, never by hand; droppings damped down and removed with RPE. • Welfare with hot water for washing before eating; cuts covered and any needle injury reported at once. | 1 x 3 = 3 Low |  |
| Manual handling and sharp materials | Operatives stripping out glass, timber, plasterboard, ceilings and fixtures. | 4 x 2 = 8 Medium | • Strip out arisings moved by skip, chute or machine rather than carried, with short carrying routes. • Glass removed early and handled with cut-resistant gloves and sleeves; nails knocked down or removed. | 2 x 2 = 4 Low |  |
| Basements, voids and below-ground work | Operatives and plant near cellars, voids, tanks and open excavations left by the demolition. | 3 x 5 = 15 High | • Basements, voids, tanks and old shafts identified in the survey and fenced, covered or filled as the work reaches them. • Basement walls propped to the design where they hold up the ground or a road behind them. • Plant kept back from the edge of voids and excavations. | 1 x 5 = 5 Medium |  |
| Unauthorised entry out of hours | Trespassers and children entering a part-demolished structure at night or at weekends. | 3 x 5 = 15 High | • Hoarding secured, ladders removed, and plant immobilised at the end of each day. • Structure left stable at the end of every shift, with no part-cut members or free-standing walls left unsupported. | 1 x 5 = 5 Medium |  |

MHSWR regulation 3(6): with five or more employees, record the significant findings.

EQUIPMENT

# Plant, equipment and PPE

| Item | Checks and certificates | Operator competence |
| --- | --- | --- |
| High reach or demolition excavator | Daily check; cab guard fitted; attachment change checked. | CPCS or NPORS card |
| Remote controlled demolition machine | Daily check; controls and emergency stop tested. | Trained operator |
| Breakers, saws and hand tools | Checked before use; vibration data known for planning. | Trained operative |
| Crusher and screen | Daily check; guards and emergency stops working. | Trained operator |
| Oxy-fuel cutting set | Hoses, regulators and flashback arrestors checked before use. | Trained operative |
| Dust suppression | Water supply and spray units working before demolition starts. | Supervisor |

## Personal protective equipment

PPE is the last line of defence, after the controls above. Specify the type, not just the item.

| Item | Needed | Type and specification |
| --- | --- | --- |
| Safety helmet | Yes | Worn at all times on site |
| Safety footwear | Yes | Midsole protection against nails |
| High-visibility clothing | Yes | To site standard |
| Gloves | Yes | Cut-resistant for glass; grip gloves |
| Eye protection | Yes | Breaking, cutting and near machines |
| Hearing protection | Yes | Breakers, saws, crushers |
| Respiratory protection | Yes | FFP3, face fit tested, for dust |
| Harness or restraint |  |  |

CONTROLS

# Substances, permits, emergencies and the environment

## Hazardous substances

| Substance | COSHH assessment reference | Controls |
| --- | --- | --- |
| Silica and demolition dust | [ref] | Water suppression; sealed cabs; FFP3 RPE, face fit tested, outside a cab. |
| Asbestos | [survey ref] | Removed before demolition, licensed contractor where licensable; suspect finds stop work. |
| Lead paint fume from hot cutting | [ref] | Paint checked before burning; cold cutting or shears where lead is found. |
| Bird droppings and biological hazards | [ref] | Damped down and removed with RPE and gloves; wash before eating. |

## Permits and plans

| Permit or plan | Needed | Reference |
| --- | --- | --- |
| Hot work permit | Yes |  |
| Permit to work | Yes |  |
| Permit to dig or break ground | Yes |  |
| Confined space entry |  |  |
| Lift plan | Yes |  |
| Temporary works design | Yes |  |

## Emergency and rescue

If the structure moves or cracks unexpectedly, stop work, clear the exclusion zone and call the engineer. For a service strike or gas smell, stop, move everyone clear and call the owner and the emergency services. First aider and kit on site, and the nearest accident and emergency department named on the briefing.

## Environment and neighbours

Dust suppressed at the point of work and on crushers. Noise and vibration kept to the hours agreed with the local authority. Materials sorted for reuse and recycling, with a waste transfer note for every load. Spill kit with the plant. Drains protected from slurry.

BEFORE DEMOLITION STARTS

# Pre-demolition check

Complete before any structure is touched. Demolition does not start until every line is checked or marked not applicable.

| Check | What is needed | Result or reference | Checked by and time |
| --- | --- | --- | --- |
| Asbestos survey | Refurbishment and demolition survey of the whole structure, read and briefed |  |  |
| Asbestos removal | Removal complete; clearance certificates for each area of licensed work |  |  |
| Structural survey | Form, load paths and condition assessed; engineer advice where needed |  |  |
| Written method | Sequence of demolition recorded and approved before work begins |  |  |
| Local authority | England and Wales: section 80 notice given; section 81 reply received, or 6 weeks passed |  |  |
| Electricity | Disconnected by the owner, written confirmation, proven dead |  |  |
| Gas and water | Disconnected by the owners, written confirmation on file |  |  |
| Adjoining buildings | Condition recorded; party walls assessed and propped where needed |  |  |
| Zones | Exclusion and debris zones set out and barriered; public kept out |  |  |
| Overhead lines | Identified; diverted, switched off or protected by goal posts |  |  |
| Hazardous finds | Sharps, droppings, chemicals and fuel found and dealt with |  |  |
| Emergency plan | Collapse, fire and service strike arrangements briefed |  |  |

CDM 2015 regulations 19 and 20; Control of Asbestos Regulations 2012 regulations 5 and 7; HSE HSG264; Building Act 1984 section 80.

BRIEFING RECORD

# Briefed before the work starts

Each person signs against this revision, on the day, before starting. Anyone joining later is briefed before they join the work. When the RAMS is revised, everyone is briefed again.

| Briefed by | Revision briefed | Date and time |
| --- | --- | --- |

| Name | Company | Card or ticket number | Signature | Date |
| --- | --- | --- | --- | --- |