[Your logo]

Risk assessment and method statement

# Landscaping and fencing RAMS

Fencing, hard and soft landscaping. Pre-filled with typical hazards, controls and sequence: edit it for this site.

Management of Health and Safety at Work Regulations 1999, regulation 3. PUWER 1998 and COSHH 2002.

| Detail | Entry |
| --- | --- |
| Project |  |
| Site address |  |
| Activity | Fencing, hard and soft landscaping |
| RAMS reference |  |
| Contractor carrying out the work |  |
| Principal contractor |  |
| Prepared by |  |
| Approved by |  |
| Revision |  |
| Date |  |

Revision record. Reissue and rebrief when the sequence, the programme, the team or the conditions change.

| Rev | Date | What changed | By |
| --- | --- | --- | --- |

THE WORK

# The activity, the place and the people

## Scope of the activity

Installation of [fence type] fencing along the line shown on [drawing ref], including posts, rails, panels and gates. Hard landscaping: [paving / kerbs / edgings] to [area]. Soft landscaping and maintenance: [turf / planting / mowing / hedge cutting]. Herbicide treatment: [yes / no]. Not included: [list any exclusions].

## Location and access

Site access via [gate]. Materials delivered to [area] and stacked close to the work. Plant route [route], kept apart from pedestrian routes by barriers. Public footpath or road on [boundary] kept open or diverted. Working hours [08:00 to 17:00].

## Interfaces with other trades

Other trades near the fence line or landscaping area: [trades], kept outside the work area. Service owners: [names], for plans and any work near their assets. Client or site contact for gates, keys and occupied areas: [name].

## Documents this RAMS relies on

Drawings and fence specification [refs]. Service plans from each owner [refs]. Permit to dig where required. COSHH assessments and product labels for herbicides, cement and fuel. Traffic management plan [ref]. Construction phase plan.

## Programme

| Start date | Duration | Working hours |
| --- | --- | --- |

## The team, and their competence

| Role | Name | Card, ticket or training needed | Contact |
| --- | --- | --- | --- |
| Operatives |  | Trained on each machine they use, including mowers, strimmers, augers and post drivers. |  |
| Cut-off saw users |  | Trained on the saw and in selecting and fitting discs. |  |
| Pesticide users |  | A specified certificate for the method of application, or under direct supervision while training. |  |
| Plant operators |  | CPCS or NPORS card, or equivalent training, for dumpers, excavators and telehandlers on site. |  |
| Locator operator |  | Trained in the cable locator and signal generator and their limits. |  |
| Supervisor |  | Experienced in the work, on site or in regular contact throughout. |  |

METHOD STATEMENT

# The sequence of work

The order the team will work in on the day, with the controls built into each stage. If the sequence changes on site, revise the RAMS and brief it again.

| Stage | What is done, and how | Controls at this stage | By |
| --- | --- | --- | --- |
| Before the day | Survey the site, confirm the drawings and fence line, and obtain service plans. Note overhead lines, slopes, water, public routes and any work on the highway. Book plant and deliveries. | Service plans in date; COSHH assessments for each product; RAMS approved by the principal contractor or client. | Contracts manager |
| Arrival and briefing | Sign in, attend the site induction, then brief the team on this RAMS, the services found, the exclusion zones and the first aid arrangements. Check plant and tools. | Briefing signed against this revision; cards and pesticide certificates checked; pre-use checks recorded. | Supervisor |
| Set up the work area | Barrier and sign the work area, agree delivery and plant routes, and set up the cutting area with water for suppression. | Public routes kept open or diverted; banksman for vehicle movements; goal posts under any overhead line. | Supervisor |
| Locate and mark services | Scan the fence line and dig areas with the cable locator and signal generator, mark the services found, and dig trial holes by hand where the line crosses them. | Permit to dig where the site requires one; marks in paint or wooden pegs, no steel pins. | Locator operator |
| Clear and set out | Walk the area for stones, glass and wire, clear vegetation, and set out the fence line or landscaping levels with string lines and pegs. | Brushcutter and strimmer zones kept clear of others; visors worn. | Operatives |
| Posts and fence line | Dig, auger or drive the posts, set them in concrete or postcrete where specified, then fit rails, panels, mesh or boards and any gates. | No post over a marked service; nobody steadies a post under the rammer; holes filled or covered the same day. | Fencers |
| Hard landscaping | Excavate and prepare the sub-base, compact it, then lay slabs, blocks, kerbs and edgings, cutting units as needed. | Water suppression and RPE for cutting; lifters for heavy units; vibration exposure planned for compaction. | Operatives |
| Soft landscaping and mowing | Spread topsoil, lay turf, plant, and carry out mowing, strimming and hedge cutting to the specification. | Slopes within mower limits; blades stopped and key out before clearing blockages; eye and hearing protection. | Operatives |
| Herbicide treatment | Where in scope, mix and apply herbicide to the label, then record the product, rate, area and weather. | Certificated operator or supervised trainee; label PPE; no drift; public kept off treated areas. | Pesticide operator |
| Clear the site | Remove arisings, offcuts and packaging, wash down tools, take down barriers and walk the finished work with the client. | Washings kept out of drains; containers and waste removed; area handed back before the team leaves. | Supervisor |

RISK ASSESSMENT

# Hazards, controls and the risk that remains

Each significant hazard rated before and after its controls, as likelihood (1 to 5) times severity (1 to 5). Bands: 1 to 4 low, 5 to 12 medium, 15 to 25 high. The ratings are typical: check them for this site.

| Hazard | Who could be harmed, and how | Risk before | Controls | Risk after | By |
| --- | --- | --- | --- | --- | --- |
| Striking buried services with posts | Fencers digging, augering or driving posts, and anyone nearby, from shock, burns, gas escape or flooding. | 4 x 5 = 20 High | • Service plans checked, the fence line scanned with a cable locator and signal generator, and services marked before any post goes in. • Trial holes dug by hand to confirm position and depth where the line crosses or runs near a service. • No post driven, augered or pinned over a marked line; marks in paint or wooden pegs, never steel pins. | 1 x 5 = 5 Medium | Supervisor |
| Overhead power lines | Operators of post drivers, augers, tippers and telehandlers, and anyone carrying long rails or poles. | 2 x 5 = 10 Medium | • Lines identified at the survey; work within 10 metres, measured at ground level from below the nearest wire, planned first and the line owner consulted, as HSE GS6 advises. • Where lines stay live, ground barriers at least 6 metres either side keep plant away, with goal posts at any crossing. • Long rails, posts and poles carried horizontally and close to the ground near lines. | 1 x 5 = 5 Medium |  |
| Machine-mounted post drivers | The operator and the person setting the post, from crushing, flying splinters and the post kicking out. | 3 x 4 = 12 Medium | • Operated only by someone trained on that driver, with guards and controls checked before use. • Post set in a pilot hole or held in the driver's post holder; nobody steadies a post by hand under the rammer. • Everyone else kept clear of the driver while it works; eye and hearing protection worn. | 1 x 4 = 4 Low | Operator |
| Powered augers: torque and entanglement | Operators of hand-held and machine-mounted augers when the bit jams on roots or stone. | 3 x 3 = 9 Medium | • Two-person or frame-mounted augers for larger holes, with a torque reaction bar where the auger has one. • Close-fitting clothing, no loose cuffs or cords; engine stopped before freeing a jammed bit. • Ground checked for roots, stone and services before boring. | 2 x 3 = 6 Medium |  |
| Hand-arm vibration | Operatives using hand-held post drivers, breakers, strimmers, hedge trimmers and plate compactors. | 4 x 3 = 12 Medium | • Machine-mounted drivers or lower vibration tools chosen where practicable, and tools maintained to the manufacturer's schedule. • Trigger time planned per tool and per person against the exposure action value of 2.5 m/s2 A(8). • Health surveillance where the assessment shows a risk; tingling or numbness reported at once. | 2 x 3 = 6 Medium |  |
| Noise from saws, drivers and mowers | Operatives using cut-off saws, post drivers, mowers, strimmers and blowers, and people working beside them. | 4 x 3 = 12 Medium | • Quieter equipment and methods chosen where practicable, and exhausts and silencers kept in good order. • Hearing protection worn at or above 85 dB(A), the upper exposure action value, and available on request from 80 dB(A). • Others kept back from noisy work where they can be. | 2 x 3 = 6 Medium |  |
| Cut-off saws: contact and kickback | Operatives cutting slabs, blocks, kerbs and setts, and anyone close to the cut. | 3 x 5 = 15 High | • Operated only by people trained on the saw and in fitting discs; disc rated for the saw speed and the material. • Guard set correctly, both hands on the saw, no cutting above shoulder height, and the work held firm on a stand or block. • Saw stopped before being put down; others kept clear of the cutting line. | 1 x 5 = 5 Medium |  |
| Silica dust from cutting paving | Operatives cutting concrete or stone slabs, blocks, kerbs and setts, and people working nearby. | 4 x 4 = 16 High | • Cuts reduced at the layout stage, a block splitter used where it will do, or units cut off site. • Water suppression on every cut-off saw cut, with the water supply checked before starting. • RPE with an assigned protection factor of 20, such as an FFP3 mask, face fit tested and worn with water suppression. | 2 x 4 = 8 Medium |  |
| Manual handling of slabs, kerbs and posts | Operatives moving paving slabs, kerbs, posts, panels, rolls of turf and bags of aggregate. | 4 x 3 = 12 Medium | • Slab and kerb lifters, vacuum lifters or plant used for heavy units rather than lifting by hand. • Deliveries placed close to where they are laid, so carrying distances stay short. • Heavy posts and panels handled by two people; knee pads for laying work. | 2 x 3 = 6 Medium |  |
| Ride-on mowers overturning | Ride-on mower operators on slopes, banks and the edges of ditches, ponds and retaining walls. | 3 x 5 = 15 High | • Slopes assessed before mowing and kept within the manufacturer's limits; steeper banks cut by pedestrian or remote mower. • Roll-over protection kept up and the seat belt worn where the mower has them. • Edges of drops and water marked or cut by hand from a safe distance. | 1 x 5 = 5 Medium |  |
| Mower blades and blockages | Operators clearing grass from decks and chutes, and people near the mower. | 3 x 4 = 12 Medium | • Blades stopped, engine off and key removed before clearing a blockage or reaching under the deck. • Guards and collection boxes kept in place; no mowing with a damaged guard. | 1 x 4 = 4 Low |  |
| Thrown stones from strimmers and mowers | Operators, colleagues and the public near strimmers, brushcutters and mowers. | 3 x 3 = 9 Medium | • Area walked first and stones, glass and wire removed. • Visor or eye protection worn, guards fitted, and cutting stopped when anyone comes close. • Work near roads, windows and parked cars planned so debris is thrown away from them. | 2 x 3 = 6 Medium |  |
| Herbicides and pesticides | Operators mixing and spraying, the public, pets and nearby watercourses. | 3 x 4 = 12 Medium | • COSHH assessment and the label read before use; only professional products the operator is certificated for, or used under direct supervision while training. • Label PPE worn; mixing and filling away from drains and water, with a spill kit. • Use kept as low as reasonably practicable in public areas, school grounds and parks; no spraying in wind that causes drift. | 1 x 4 = 4 Low | Pesticide operator |
| Wet cement and postcrete | Operatives setting posts and laying bedding mortar and concrete. | 3 x 3 = 9 Medium | • Waterproof gloves suitable for alkalis, and cement washed off the skin straight away. • Eye protection when mixing or pouring dry mix; eyewash in the van. | 1 x 3 = 3 Low |  |
| Petrol, fuelling and fire | Operators refuelling mowers, saws, augers and drivers, and anyone near the fuel store. | 2 x 4 = 8 Medium | • Fuel kept in approved, labelled containers, away from ignition sources and secured in the van. • Refuelling with the engine stopped and cool, on open ground, with spills wiped up before starting. | 1 x 4 = 4 Low |  |
| Plant, vehicles and the public | Operatives, other trades, the public and road users near plant, deliveries and work on the highway edge. | 3 x 5 = 15 High | • Work area barriered and signed, with pedestrian routes kept open or diverted. • Banksman for reversing and for plant near people; dumpers with roll-over protection and seat belts worn. • Work on or beside a road set out under a traffic management plan agreed with the highway authority. | 1 x 5 = 5 Medium |  |
| Open post holes, slopes and uneven ground | Operatives, other trades and the public walking near the fence line or the landscaping area. | 3 x 3 = 9 Medium | • Post holes filled the same day or covered and marked; nothing left open at the end of the shift. • Tools, offcuts and materials kept off walkways; steep or wet ground walked before plant goes on it. | 1 x 3 = 3 Low |  |
| Needles, ticks and rat urine | Operatives clearing litter, scrub and long grass in parks, verges and ground near water. | 3 x 3 = 9 Medium | • Area walked first; needles picked up only with a litter picker into a sharps box, never by hand. • Gloves worn, cuts covered, and hands washed before eating, drinking or smoking, especially near water or rats. • Long sleeves and trousers in long grass and scrub, with a check for ticks after work. | 1 x 3 = 3 Low |  |

MHSWR regulation 3(6): with five or more employees, record the significant findings.

EQUIPMENT

# Plant, equipment and PPE

| Item | Checks and certificates | Operator competence |
| --- | --- | --- |
| Post driver, hand-held or mounted | Pre-use check; guards and controls working; vibration data known. | Trained operator |
| Powered auger | Pre-use check; bit secure; torque bar fitted where provided. | Trained operator |
| Cut-off saw | Disc rated and undamaged; guard set; water supply working. | Trained in the saw |
| Ride-on and pedestrian mowers | Daily check; guards, blade brake and seat switch working. | Trained operator |
| Strimmers, brushcutters, hedge trimmers | Guards fitted; line or blade sound; harness adjusted. | Trained operator |
| Knapsack or pedestrian sprayer | Checked for leaks; nozzles and calibration suit the label. | Certificated operator |
| Cable locator and signal generator | Calibration or self-test in date; checked before use. | Trained operator |

## Personal protective equipment

PPE is the last line of defence, after the controls above. Specify the type, not just the item.

| Item | Needed | Type and specification |
| --- | --- | --- |
| Safety helmet |  |  |
| Safety footwear | Yes | Toe protection, grip sole, for wet ground |
| High-visibility clothing | Yes | Near plant, vehicles and the highway |
| Gloves | Yes | Grip; waterproof for cement; label gloves |
| Eye protection | Yes | Strimming, cutting, driving posts, mixing |
| Hearing protection | Yes | Saws, drivers, mowers, strimmers |
| Respiratory protection | Yes | FFP3, face fit tested, for cutting |
| Harness or restraint |  |  |

CONTROLS

# Substances, permits, emergencies and the environment

## Hazardous substances

| Substance | COSHH assessment reference | Controls |
| --- | --- | --- |
| Herbicides | [product and ref] | Certificated operator; label PPE; no drift; away from drains and water. |
| Wet cement and postcrete | [ref] | Alkali-resistant gloves and eye protection; wash skin straight away. |
| Silica dust from cutting | [ref] | Fewer cuts; water suppression; FFP3 RPE, face fit tested. |
| Petrol and two-stroke fuel | [ref] | Approved containers; refuel with engine off and cool; no ignition sources. |

## Permits and plans

| Permit or plan | Needed | Reference |
| --- | --- | --- |
| Hot work permit |  |  |
| Permit to work | Yes |  |
| Permit to dig or break ground | Yes |  |
| Confined space entry |  |  |
| Lift plan |  |  |
| Temporary works design |  |  |

## Emergency and rescue

First aider and first aid kit on site or in the van, with eyewash for cement and chemicals and the nearest accident and emergency department named on the briefing. For a service strike: stop, move everyone clear, call the service owner and the emergency services, and do not touch damaged cables or pipes.

## Environment and neighbours

Slurry from wet cutting and cement washings kept out of drains and watercourses. Herbicide mixed and applied away from water, with a spill kit. Green waste, offcuts and packaging removed from site. Nesting birds checked before hedge or shrub clearance.

BRIEFING RECORD

# Briefed before the work starts

Each person signs against this revision, on the day, before starting. Anyone joining later is briefed before they join the work. When the RAMS is revised, everyone is briefed again.

| Briefed by | Revision briefed | Date and time |
| --- | --- | --- |

| Name | Company | Card or ticket number | Signature | Date |
| --- | --- | --- | --- | --- |