[Your logo]

Risk assessment and method statement

# Noise and vibration risk assessment

Work with noisy and vibrating tools. Pre-filled with typical hazards, controls and sequence: edit it for this site.

Management of Health and Safety at Work Regulations 1999, regulation 3. Control of Noise at Work and Vibration at Work Regulations 2005.

| Detail | Entry |
| --- | --- |
| Project |  |
| Site address |  |
| Activity | Work with noisy and vibrating tools |
| RAMS reference |  |
| Contractor carrying out the work |  |
| Principal contractor |  |
| Prepared by |  |
| Approved by |  |
| Revision |  |
| Date |  |

Revision record. Reissue and rebrief when the sequence, the programme, the team or the conditions change.

| Rev | Date | What changed | By |
| --- | --- | --- | --- |

THE WORK

# The activity, the place and the people

## Scope of the activity

Use of noisy and vibrating tools and plant on [site or job]: [list tasks, for example breaking out concrete, chasing walls, cutting kerbs, compacting sub-base]. Each tool is listed on the exposure calculator page with its vibration magnitude or noise level, time limit and controls. Not included: [list exclusions].

## Location and access

Site access via [gate]. Noisy work at [location], marked as a hearing protection zone and screened from other work areas. Generators and compressors sited at [location]. Hours for noisy work [08:00 to 17:00].

## Interfaces with other trades

Other trades and occupants near noisy work: [names]. Breaking and other noisy work is programmed for when others are clear, and the hearing protection zone is kept as small as practicable.

## Documents this RAMS relies on

Tool vibration and noise data with sources [ref]; exposure calculations on the calculator page; HSE exposure calculators; health surveillance arrangements; tool maintenance records; construction phase plan.

## Programme

| Start date | Duration | Working hours |
| --- | --- | --- |

## The team, and their competence

| Role | Name | Card, ticket or training needed | Contact |
| --- | --- | --- | --- |
| Assessor |  | Understands the action values, the points system and the tools in use, and uses the HSE calculators. |  |
| Supervisor |  | Plans trigger times and rotation, and checks they are kept on the day. |  |
| Operatives |  | Trained to use and check their tools, fit hearing protection and report symptoms. |  |
| Occupational health |  | A competent provider for hearing checks and HAVS health surveillance. |  |
| Plant drivers |  | Trained for the plant they drive, and briefed on seat adjustment and haul routes. |  |

METHOD STATEMENT

# The sequence of work

The order the team will work in on the day, with the controls built into each stage. If the sequence changes on site, revise the RAMS and brief it again.

| Stage | What is done, and how | Controls at this stage | By |
| --- | --- | --- | --- |
| List the tools and tasks | List every tool and machine the team will use. Record its vibration magnitude and noise level for real use, from the supplier, HSE typical figures or measurement. | Each figure recorded with its source on the exposure calculator page. | Supervisor |
| Find the trigger time | Find out how long each person has each tool running in their hands, or is in the noise, each day. Watch the work: trigger time is often far less than time on the job. | Times based on observation, not estimates alone. | Supervisor |
| Work out the daily dose | Add up each person's daily vibration and noise exposure across all their tools. Use the exposure points system or the HSE calculators. | Vibration: 100 points action value, 400 limit. Noise: 80 and 85 dB(A) action values. | Assessor |
| Change the method or tool | Where exposure reaches an action value, set a programme of controls: other methods, quieter or low-vibration tools, and better maintenance. | Hearing protection does not count as part of the noise programme required at 85 dB(A). | Contracts manager |
| Limit time and rotate | Set a maximum trigger time for each tool. Rotate tasks so each person stays under the action value where reasonably practicable, and never over the limit value. | Daily time limits written on the briefing; supervisor checks they are kept. | Supervisor |
| Mark hearing protection zones | Mark areas where exposure could reach 85 dB(A), put up signs, and provide hearing protection matched to the noise. | Signs in place; protection worn by everyone in the zone. | Supervisor |
| Brief the team | Brief the assessment, the time limits, the tool checks, how to fit hearing protection, and the symptoms to report. | Briefing signed against this revision. | Supervisor |
| Check tools daily | Check tools before use: sharp bits and discs, handles and mounts intact, and no unusual vibration or noise. Take faulty tools out of use. | Faulty tools tagged and kept out of use until repaired. | Operatives |
| Health surveillance | Enrol exposed operatives in hearing checks and HAVS health surveillance as the assessment requires, and act on the results. | Health records kept; RIDDOR report made for a diagnosed HAVS or carpal tunnel case. | Contracts manager |
| Review | Review the assessment when tools, methods or the team change, after a health surveillance finding, and at set intervals. | Review dated and signed; new figures briefed to the team. | Assessor |

RISK ASSESSMENT

# Hazards, controls and the risk that remains

Each significant hazard rated before and after its controls, as likelihood (1 to 5) times severity (1 to 5). Bands: 1 to 4 low, 5 to 12 medium, 15 to 25 high. The ratings are typical: check them for this site.

| Hazard | Who could be harmed, and how | Risk before | Controls | Risk after | By |
| --- | --- | --- | --- | --- | --- |
| Vibration from breakers and hammers | Operatives using breakers, demolition hammers, rotary hammers and scabblers. | 4 x 4 = 16 High | • Method changed where possible, for example bursting, core drilling or a remote-controlled machine in place of hand breaking. • Low-vibration tools chosen, judged on the vibration figure for real use, not just the declared value. • Trigger time limited and tasks rotated so each person stays under the action value where reasonably practicable. | 2 x 4 = 8 Medium | Supervisor |
| Vibration from grinders, drills and saws | Operatives using angle grinders, hammer drills, cut-off saws, reciprocating saws and sanders. | 4 x 4 = 16 High | • Exposure points worked out for each person each day, from the tool magnitude and trigger time. • Discs, bits and blades kept sharp and of the right type, and tools serviced to the maker's schedule. • Anti-vibration handles and mounts kept in good repair. | 2 x 4 = 8 Medium |  |
| Vibration from compactors and rammers | Groundworkers using plate compactors, trench rammers and pedestrian rollers. | 4 x 4 = 16 High | • Vibration-reduced compactors chosen: HSE typical figures are 2 to 7 m/s², against 9 to 22 m/s² for older types. • Use time controlled through the daily points total, with tasks rotated on long runs. | 2 x 4 = 8 Medium |  |
| Combined exposure from several tools | Operatives who use more than one noisy or vibrating tool in a day. | 3 x 4 = 12 Medium | • Points from every tool added together for the day, for vibration and for noise. • Work planned so the daily total stays under the action values, by rotating tasks or spreading heavy work across days. | 2 x 4 = 8 Medium | Supervisor |
| Cold and wet hands | Operatives using vibrating tools outdoors in cold or wet weather. | 3 x 4 = 12 Medium | • Warm, dry gloves and clothing provided, as the vibration regulations require protection from cold and damp to be considered. • Warm welfare for breaks, and hands warmed before starting work. | 2 x 4 = 8 Medium |  |
| Early HAVS symptoms missed | Operatives likely to be exposed at or above the vibration action value. | 3 x 4 = 12 Medium | • Health surveillance for everyone likely to be exposed at or above the action value, by a competent occupational health provider. • Operatives briefed to report tingling, numbness or fingers going white straight away. • Any diagnosed HAVS or carpal tunnel syndrome reported under RIDDOR, and the person's work reviewed. | 1 x 4 = 4 Low | Contracts manager |
| Noise from breakers, saws and grinders | Operatives using noisy tools and anyone working nearby. | 4 x 4 = 16 High | • Quieter methods and tools chosen first, such as bursting or diamond drilling in place of breaking. • Noisy work screened or sited away from others, and timed for when others are clear. • Hearing protection that brings the level at the ear below 85 dB(A) but not under 70 dB(A), worn the whole time in the noise. | 2 x 4 = 8 Medium |  |
| Noise from generators and compressors | Operatives and others working near site plant. | 3 x 4 = 12 Medium | • Plant sited away from work areas, with acoustic covers closed and silencers fitted. • Plant maintained so it runs no louder than it should, and switched off when idle. | 2 x 4 = 8 Medium |  |
| Peak noise from impact work | Operatives using cartridge tools and nail guns, or hammering steel. | 3 x 4 = 12 Medium | • Peak noise assessed as well as the daily dose: action values of 135 and 137 dB(C), limit 140 dB(C). • Hearing protection chosen for impact noise, and others kept away from the work. | 2 x 4 = 8 Medium |  |
| Hearing protection that does not protect | Wearers with poorly fitted plugs, worn seals, or protection taken off in the noise. | 3 x 4 = 12 Medium | • Protection chosen with the wearers, matched to the noise and compatible with helmets and eyewear. • Wearers shown how to fit it, and checked; disposable plugs not reused. • Seals and headbands inspected, and damaged protectors replaced. | 2 x 4 = 8 Medium |  |
| Hearing protection zones not marked | Anyone entering an area where exposure could reach 85 dB(A). | 3 x 4 = 12 Medium | • Zone marked with the mandatory ear protection sign, and entry restricted where practicable. • Only people wearing hearing protection go into the zone. | 1 x 4 = 4 Low |  |
| Hearing damage not picked up | Operatives regularly exposed above the upper action value. | 3 x 4 = 12 Medium | • Hearing checks where the assessment shows a risk to hearing, with health records kept. • If damage is found, the person is told, the assessment reviewed and quieter work considered. | 2 x 4 = 8 Medium |  |
| Alarms and shouts not heard in the noise | Operatives wearing hearing protection near moving plant and vehicles. | 3 x 5 = 15 High | • Banksman and visual signals for vehicle movements in noisy areas. • Protection chosen to avoid over-protection, so reversing alarms and shouts can still be heard. | 1 x 5 = 5 Medium |  |
| Whole-body vibration from plant | Drivers of dumpers, excavators and rollers, especially on rough ground. | 3 x 3 = 9 Medium | • Haul routes kept level and maintained, with speed suited to the ground. • Seats adjusted to the driver and in good repair, and suspension maintained. • Exposure kept under the action value of 0.5 m/s² A(8), with breaks and rotation on long shifts. | 2 x 3 = 6 Medium |  |

MHSWR regulation 3(6): with five or more employees, record the significant findings.

EQUIPMENT

# Plant, equipment and PPE

| Item | Checks and certificates | Operator competence |
| --- | --- | --- |
| Breakers and hammers | Vibration-reduced type; handles and mounts checked; bits sharp. | Trained operative |
| Grinders and cut-off saws | Right disc, sharp and undamaged; guard in place. | Abrasive wheels trained |
| Plate compactors | Vibration-reduced type; handle mounts checked before use. | Trained operative |
| Generators and compressors | Acoustic covers closed; silencers fitted; serviced. | Competent person |
| Dumpers and rollers | Seat, suspension and tyres checked daily. | Trained driver |

## Personal protective equipment

PPE is the last line of defence, after the controls above. Specify the type, not just the item.

| Item | Needed | Type and specification |
| --- | --- | --- |
| Safety helmet | Yes | To site standard |
| Safety footwear | Yes | Safety boots with protective toecaps |
| High-visibility clothing | Yes | To site standard |
| Gloves | Yes | Warm and dry, for grip and warmth |
| Eye protection | Yes | For breaking, grinding and cutting |
| Hearing protection | Yes | Matched to the noise; worn in the zone |
| Respiratory protection |  |  |
| Harness or restraint |  |  |

CONTROLS

# Substances, permits, emergencies and the environment

## Hazardous substances

| Substance | COSHH assessment reference | Controls |
| --- | --- | --- |
| Silica dust from breaking and cutting | [COSHH ref] | Water suppression or on-tool extraction, FFP3 fit-tested |

## Permits and plans

| Permit or plan | Needed | Reference |
| --- | --- | --- |
| Hot work permit |  |  |
| Permit to work | Yes |  |
| Permit to dig or break ground | Yes |  |
| Confined space entry |  |  |
| Lift plan |  |  |
| Temporary works design |  |  |

## Emergency and rescue

First aid and the site emergency procedure for any injury. Operatives report tingling, numbness, fingers going white or sudden changes in hearing to the supervisor. The supervisor reviews their exposure and refers them to occupational health. A diagnosed HAVS or carpal tunnel case is reported under RIDDOR.

## Environment and neighbours

Noisy work kept to agreed hours. Plant sited away from neighbours, with acoustic covers closed and engines off when idle. Neighbours told about noisy work in advance where the client requires it.

EXPOSURE CALCULATOR

# Daily noise and vibration exposure

Vibration points per hour are 2 x magnitude x magnitude: 100 points is the action value, 400 the limit. Noise: 85 dB(A) for 8 hours is 100 points, and 80 dB(A) is 32.

| Tool or task | Magnitude in m/s² or dB(A), and its source | Time per day | Points or dose | Action needed |
| --- | --- | --- | --- | --- |
| Example: angle grinder, 100 to 180mm, vibration | 7 m/s² (example, use the manufacturer's or measured figure) | 1 hour trigger time | 98 points | Under 100 on its own; add the other tools used that day |
| Example: hammer drill into masonry | 11 m/s² (example, use the manufacturer's or measured figure) | 20 minutes | 81 points | Running total 179 points: above the action value |
| Example: vibration-reduced breaker | 14 m/s² (example, use the manufacturer's or measured figure) | 15 minutes | 98 points | Day total 277 points: above action value, under limit. Rotate tasks; health surveillance |
| Example: angle grinder, noise | 100 dB(A) (example, use the manufacturer's or measured figure) | 1 hour | 395 noise points, about 91 dB(A) daily | Above 85 dB(A): hearing protection zone, protection worn, quieter method sought |

Control of Noise at Work and Control of Vibration at Work Regulations 2005, reg 4; HSE calculators

BRIEFING RECORD

# Briefed before the work starts

Each person signs against this revision, on the day, before starting. Anyone joining later is briefed before they join the work. When the RAMS is revised, everyone is briefed again.

| Briefed by | Revision briefed | Date and time |
| --- | --- | --- |

| Name | Company | Card or ticket number | Signature | Date |
| --- | --- | --- | --- | --- |