[Your logo]

# Daily vehicle check

The driver's walkaround check before every journey, a week per sheet.

| Vehicle registration | Make and model | Week commencing | Driver or drivers |
| --- | --- | --- | --- |

Each day: tick each item when it has been checked and is fine. Mark a cross for a defect and record it in the defect report. Tick nil defects when nothing was found. A dangerous defect is reported at once and the vehicle is not used until it is fixed.

| Check | Mon | Tue | Wed | Thu | Fri | Sat | Sun |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Front view and mirrors: glass clear and undamaged, mirrors and cameras in place |  |  |  |  |  |  |  |
| Washers and wipers: work correctly, in good condition |  |  |  |  |  |  |  |
| Dashboard warning lights: braking, airbag, steering and stability lights |  |  |  |  |  |  |  |
| Steering: moves freely, no jamming or excessive play |  |  |  |  |  |  |  |
| Horn: works when operated |  |  |  |  |  |  |  |
| Brakes: footbrake and handbrake, no excessive travel |  |  |  |  |  |  |  |
| Seats and seat belts: secure, no cuts or fraying, retract fully |  |  |  |  |  |  |  |
| Lights and indicators: all work, lenses fitted, clean, right colour |  |  |  |  |  |  |  |
| Fluids, fuel and oil: levels correct, filler cap secure, no leaks |  |  |  |  |  |  |  |
| Battery: secure, in good condition, not leaking |  |  |  |  |  |  |  |
| Bodywork and doors: doors secure, no sharp edges or loose panels |  |  |  |  |  |  |  |
| Tyres and wheels: legal tread, inflated, no cuts, wheels secure |  |  |  |  |  |  |  |
| Load: secured, right restraints, bulkhead and floor sound |  |  |  |  |  |  |  |
| AdBlue: enough for the journey, diesel vehicles |  |  |  |  |  |  |  |
| Exhaust: no leaks, excessive smoke or noise |  |  |  |  |  |  |  |
| Alternative fuel system: isolation switch known, no leaks |  |  |  |  |  |  |  |
| Number plates: correct, secure, easy to read |  |  |  |  |  |  |  |
| Tow bar and trailer: secure, lights connected, breakaway cable |  |  |  |  |  |  |  |
| Other equipment: tail lift, racking, ladder clamps, roof bars |  |  |  |  |  |  |  |
| Start mileage |  |  |  |  |  |  |  |
| Driver initials |  |  |  |  |  |  |  |
| Nil defects: nothing found at the check |  |  |  |  |  |  |  |

Check items from GOV.UK, Carry out van daily walkaround checks (DVSA). Tyres: at least 1.6 mm of tread across the central three-quarters, Highway Code annex 6.

DEFECT REPORT

# Defects found, and how each was put right

Record every defect found at the check or that becomes apparent during a journey. Stop somewhere safe to assess one found on the road. A dangerous defect is fixed before the vehicle goes any further. Record the registration, date, the defect or symptom, your assessment of it, your name and who it was reported to.

## Defects found

| Date and time | Item | Defect or symptom | Dangerous | Reported to | Driver |
| --- | --- | --- | --- | --- | --- |

## Defects put right

| Date | Defect | Action taken | Repaired by | Safe to use, signed |
| --- | --- | --- | --- | --- |

GOV.UK: you are responsible for making sure your van is safe to drive. Record nil defects when nothing is found. Operator licence holders should record and report every defect, and DVSA can ask for the record of a walkaround check at a roadside check. Using a vehicle in a dangerous condition is an offence (Road Traffic Act 1988 s40A).